# Model Specification 2 — Vehicle Proximity Risk Detection

Document Version:  1.0

Date:  2026-02-18

Author:  VisionSafe360 AI Research Team

Classification:  Internal Technical Specification

## 1. Model Objective

### 1.1 Problem Statement

Vehicle-pedestrian collisions are among the most severe industrial accidents. OSHA reports that "struck-by vehicle" incidents account for approximately 75% of struck-by fatalities in construction, and forklift-pedestrian collisions cause approximately 85 deaths and 34,900 serious injuries annually in the United States alone (NIOSH). In manufacturing facilities and warehouses, the co-existence of forklifts, loaders, and pedestrian workers creates persistent proximity hazard zones.

### 1.2 Detection Scope

This model must detect the spatial relationship between vehicles and workers to determine unsafe proximity in real time.

Object Classes:

| Class ID | Class Name | Description |
| --- | --- | --- |
| 0 | person | Pedestrian worker in the scene |
| 1 | forklift | Industrial forklift (all variants) |
| 2 | loader | Wheel loaders, skid steers |
| 3 | truck | Delivery trucks, dump trucks on site |
| 4 | vehicle_other | Any other motorized industrial vehicle |

Proximity Risk Levels:

| Risk Level | Definition | Threshold (approximate) |
| --- | --- | --- |
| SAFE | No vehicle-person pair within danger zone | > 5 meters |
| WARNING | Person approaching vehicle operational area | 3–5 meters |
| DANGER | Person within immediate collision risk zone | 1–3 meters |
| CRITICAL | Imminent contact / overlap of bounding boxes | < 1 meter / overlapping |

### 1.3 Technical Challenge

Unlike PPE detection (single-object classification), proximity detection is a relational reasoning task. The model must:

Detect all persons and vehicles in the frame.

Estimate the spatial distance between each person-vehicle pair.

Classify the risk level.

This requires either monocular depth estimation or geometric calibration-based distance approximation — both with inherent limitations in single-camera CCTV setups.

### 1.4 Business Impact

A single forklift-pedestrian collision can result in fatalities, six-figure legal liability, OSHA investigations, and mandatory facility shutdowns. Real-time proximity alerting can prevent the most catastrophic workplace incidents.

## 2. Industry Benchmark Research

### 2.1 Academic State-of-the-Art

| Study / System | Year | Approach | mAP@0.5 (detection) | Distance Error | Notes |
| --- | --- | --- | --- | --- | --- |
| Kim et al., "Vision-based Proximity Detection" | 2022 | YOLOv5 + geometric calibration | 89.2% | ±0.8 m | Construction zone, single camera |
| Cai et al., "Forklift-Pedestrian Collision Warning" | 2021 | SSD + depth regression head | 84.6% | ±1.2 m | Warehouse environment |
| Park et al., "Real-time Zone Monitoring" | 2023 | YOLOv8 + homography | 92.1% | ±0.5 m | Calibrated camera, flat ground |
| Martinez et al., "Monocular Distance for Safety" | 2023 | YOLO + MiDaS depth | 87.3% | ±1.5 m | Uncalibrated camera |
| Seo et al., "Deep Learning Proximity Alert" | 2022 | Faster R-CNN + projection | 86.8% | ±0.9 m | Multi-class vehicle detection |

### 2.2 Distance Estimation Method Comparison

| Method | Accuracy (distance) | Requires Calibration | Real-time | Robustness |
| --- | --- | --- | --- | --- |
| Homography (ground plane) | ±0.3–0.5 m | Yes (one-time) | Yes | High (flat ground only) |
| Bounding box height ratio | ±0.8–1.5 m | Partial | Yes | Medium |
| Monocular depth (MiDaS/DPT) | ±1.0–2.0 m | No | Moderate | Low-Medium |
| Stereo vision | ±0.1–0.3 m | Yes (stereo rig) | Yes | High |
| LiDAR fusion | ±0.05 m | Yes (sensor fusion) | Yes | Very High |

### 2.3 Target Metrics for VisionSafe360

| Metric | Minimum Acceptable | Competitive Target | Best-in-Class |
| --- | --- | --- | --- |
| mAP@0.5 (vehicle detection) | 85.0% | 91.0% | 95.0% |
| mAP@0.5 (person detection) | 90.0% | 95.0% | 98.0% |
| Distance estimation error | ±1.5 m | ±0.8 m | ±0.3 m |
| Proximity alert accuracy | 80.0% | 88.0% | 95.0% |
| False proximity alarm rate | < 5 per hour | < 2 per hour | < 0.5 per hour |

## 3. Recommended Model Architecture

### 3.1 Approach Comparison

The vehicle proximity problem can be solved with three architectural approaches:

Approach A: Detection + Geometric Distance (Recommended)

Pipeline: YOLOv8 (detect persons + vehicles) → Homography Transform → Distance Calculation → Risk Classification

Pros: Real-time, interpretable, accurate with camera calibration, lightweight.

Cons: Requires one-time camera calibration, assumes flat ground plane.

Approach B: Detection + Monocular Depth Estimation

Pipeline: YOLOv8 (detect) → MiDaS v3 / Depth Anything (depth map) → 3D Position → Distance

Pros: No calibration needed, works with any camera.

Cons: Depth estimation adds 30–100ms latency, depth accuracy is ±1.5m (insufficient for safety-critical decisions), requires second neural network.

Approach C: Detection + Tracking + Trajectory Prediction

Pipeline: YOLOv8 (detect) → ByteTrack/BoT-SORT (track) → Kalman Filter (predict trajectory) → Collision Time Estimate

Pros: Predicts future collisions before they happen, accounts for velocity.

Cons: Complex pipeline, tracking errors propagate, requires tuning.

### 3.2 Recommended Architecture

Primary: Approach A (Detection + Geometric Distance) with Approach C elements (Tracking)

Rationale:

Industrial CCTV cameras are fixed-position. One-time calibration (4 reference points on the ground plane) provides a homography matrix that converts pixel coordinates to real-world ground coordinates with ±0.3–0.5m accuracy — sufficient for safety zone classification.

Adding ByteTrack on top provides temporal consistency and velocity estimation, enabling "closing speed" analysis.

This approach requires no additional neural networks beyond YOLOv8, keeping the pipeline lightweight for edge deployment.

### 3.3 Detection Model Selection

Same YOLOv8s recommendation as Model 1 (PPE), but with different class training:

| Component | Choice | Justification |
| --- | --- | --- |
| Detection Model | YOLOv8s | Same model can detect persons AND vehicles |
| Tracker | ByteTrack | Best open-source multi-object tracker, minimal overhead |
| Distance Estimation | Homography-based | ±0.5m accuracy with calibration, zero latency |
| Risk Classifier | Rule-based | Threshold-based on calculated distance |

Note: Model 1 (PPE) and Model 2 (Vehicle Proximity) are developed independently by different team members. Each model is trained, tested, and maintained separately.

### 3.4 Independent Model Strategy

Each model is a separate YOLOv8s instance with its own classes:

Model 1 — PPE Detection (separate team member):

| Class ID | Class Name |
| --- | --- |
| 0 | person |
| 1 | helmet_on |
| 2 | helmet_off |
| 3 | vest_on |
| 4 | vest_off |

Model 2 — Vehicle Proximity (separate team member):

| Class ID | Class Name |
| --- | --- |
| 0 | person |
| 1 | forklift |
| 2 | loader |
| 3 | truck |
| 4 | vehicle_other |

Why separate models:

Two different team members work on them in parallel — merging would create blocking dependencies.

Different datasets, training schedules, and hyperparameter tuning.

Independent testing and validation — each model owner is responsible for their own metrics.

Easier debugging — each model's failure modes are isolated.

Future optimization (post-development): After both models reach their target metrics, a unified model can be explored as an optimization to reduce inference cost. This is NOT a development-phase concern.

## 4. Dataset Strategy

### 4.1 Required Dataset Size

| Phase | Images | Annotations |
| --- | --- | --- |
| Vehicle + Person Detection Set | 6,000 | ~18,000 |
| Proximity Scenarios | 4,000 | ~20,000 |
| Validation | 2,000 | ~8,000 |
| Test | 1,000 | ~4,000 |
| Calibration Images | 50–100 | Reference points only |
| Total | ~13,000 | ~50,000 |

### 4.2 Public Dataset Sources

| Dataset | Size | Classes | Quality |
| --- | --- | --- | --- |
| KITTI (vehicles) | 7,481 | Car, Van, Truck, etc. | High |
| BDD100K (vehicles) | 100,000 | Multiple vehicle types | High |
| LVIS (forklift subset) | ~500 | Forklift | Medium |
| Open Images v7 (forklift) | ~2,000 | Forklift | Medium |
| Custom warehouse footage | To collect | Forklift, Person | Required |
| COCO (person class) | 64,000 | Person | High |

Critical gap: Forklift-specific datasets are extremely limited. Custom data collection from the deployment site is mandatory.

### 4.3 Class Balance Strategy

| Class | Expected Prevalence | Balancing Strategy |
| --- | --- | --- |
| person | High (60%) | Natural prevalence, no rebalancing needed |
| forklift | Medium (20%) | Augment with synthetic overlays |
| loader | Low (5%) | Use transfer from general vehicle data |
| truck | Medium (10%) | Adequate in BDD100K/KITTI |
| vehicle_other | Low (5%) | Group rare vehicle types |

### 4.4 Data Augmentation Strategy

All augmentations from Model 1, plus:

| Augmentation | Purpose | Parameters |
| --- | --- | --- |
| Scale Jitter (vehicles) | Simulate distance variation | 0.3–2.0x |
| Partial Occlusion | Vehicle behind shelving/walls | 20–50% occlusion |
| Background Diversification | Warehouse, factory, outdoor yard | Domain-specific scenes |
| Person-near-vehicle paste | Create proximity training pairs | Controlled placement |

### 4.5 Calibration Data

For each deployment camera, collect:

4+ reference points on the ground plane with known real-world coordinates (measured with tape measure).

Record these as (pixel_x, pixel_y) → (real_x_meters, real_y_meters) pairs.

Compute homography matrix H using OpenCV findHomography.

Validate by measuring known distances in the transformed space.

## 5. Training Strategy

### 5.1 Transfer Learning Approach

Recommended: Train from COCO pre-trained weights.

COCO already contains car, truck, bus, and person classes — an excellent starting point.

Fine-tune on industrial vehicle dataset (forklift, loader, etc.).

Freeze backbone for 10 epochs, then unfreeze and fine-tune for 50–80 epochs.

This model is independent of Model 1 (PPE) — trained by a different team member.

Alternative: Train from scratch on custom dataset.

If COCO pre-trained weights perform poorly on industrial vehicles (forklifts are not in COCO), train from ImageNet-pretrained backbone.

Requires more training data but avoids domain mismatch.

### 5.2 Hyperparameter Configuration

Same as Model 1, with the following modifications:

| Parameter | Modification | Justification |
| --- | --- | --- |
| Image Size | 640 x 640 (same) | Vehicles are larger objects, 640 is sufficient |
| Anchor Sizes | Include larger anchors | Vehicles occupy more pixels than PPE items |
| Objectness Weight | Increase to 1.2 | Ensure large vehicle objects are detected |
| Class Weight | Rebalance for new classes | Prevent person class from dominating training |

### 5.3 Loss Functions

Same composite YOLOv8 loss (CIoU + BCE + DFL) as Model 1.

Custom modification:

Apply higher box loss weight for vehicle classes — accurate bounding boxes are essential because the distance calculation uses the bottom-center of the bounding box as the ground contact point.

### 5.4 Validation Strategy

Standard 80/10/10 split for detection validation.

Additional proximity validation: Create a curated set of 200 images with known person-vehicle distances (measured ground truth). Evaluate the full pipeline (detect → calibrate → distance) on this set.

Report distance estimation MAE (mean absolute error) separately from detection mAP.

## 6. Evaluation Metrics

### 6.1 Primary Metrics — Detection

| Metric | Definition | Priority |
| --- | --- | --- |
| mAP@0.5 (vehicles) | Mean AP for vehicle classes | Highest |
| mAP@0.5 (persons) | Mean AP for person class | Highest |
| Recall (vehicles) | Vehicle detection true positive rate | High |

### 6.2 Primary Metrics — Proximity

| Metric | Definition | Priority |
| --- | --- | --- |
| Distance MAE | Mean absolute error of estimated distance (m) | Highest |
| Proximity Classification Acc | Correct risk level (SAFE/WARNING/DANGER/CRITICAL) | Highest |
| Alert True Positive Rate | % of real danger situations correctly alerted | Highest |

### 6.3 Operational Metrics

| Metric | Definition | Target |
| --- | --- | --- |
| False Proximity Alarm Rate | False danger alerts per camera per hour | < 3 per hour |
| Missed Danger Rate | Real DANGER situations not detected | < 5% |
| Proximity Latency | Time from frame to proximity decision | < 500 ms |
| Tracking ID Stability | % of frames where tracked ID is consistent | > 90% |

### 6.4 Target Thresholds

| Metric | Minimum | Target | Best-in-Class |
| --- | --- | --- | --- |
| mAP@0.5 (vehicles) | 85.0% | 91.0% | 95.0% |
| mAP@0.5 (persons) | 90.0% | 95.0% | 98.0% |
| Distance MAE | ±1.5 m | ±0.8 m | ±0.3 m |
| Proximity Classification Acc | 80.0% | 88.0% | 95.0% |
| Tracking Consistency (MOTA) | 60.0% | 75.0% | 85.0% |

## 7. Deployment Strategy

### 7.0 Development vs. Production Environments

Same as Model 1: all development and testing is performed on a laptop/desktop with CUDA GPU. Edge deployment (Jetson) is the production target for a future phase.

| Phase | Hardware | Model Format | Notes |
| --- | --- | --- | --- |
| Development | Laptop/Desktop (CUDA GPU) | PyTorch (.pt) | Full development, training, testing |
| Demo/Testing | Laptop/Desktop | PyTorch or ONNX | Real-time demo with webcam or video |
| Production | NVIDIA Jetson Orin Nano | TensorRT FP16 | Final edge deployment (future phase) |

### 7.1 Export Pipeline

Development (laptop): Use PyTorch .pt directly — no conversion needed.

Production (edge): PyTorch → ONNX → TensorRT FP16

Model 2 (Vehicle Proximity) is exported independently of Model 1 (PPE). Two separate .engine files run on the edge device.

### 7.2 Proximity Computation Overhead

The distance calculation is purely geometric (matrix multiplication) and adds negligible compute:

| Component | Latency |
| --- | --- |
| YOLOv8s inference | ~30–40 ms |
| ByteTrack update | ~2–5 ms |
| Homography transform | < 1 ms |
| Distance computation | < 1 ms |
| Risk classification | < 1 ms |
| Total pipeline | ~35–50 ms |

### 7.3 Multi-Camera Calibration

Each camera requires its own homography matrix. Store calibration data as JSON per camera:

calibrations/ ├── cam_01_homography.json ├── cam_02_homography.json └── ...

Provide a calibration CLI tool that guides the operator through the 4-point ground plane mapping process.

### 7.4 Frame Scheduling

Same adaptive strategy as Model 1.

For proximity: when a vehicle AND person are both detected in the same frame, increase processing to every frame (30 FPS) until they separate by > 5 meters or one leaves the frame.

## 8. Risks & Mitigation

### 8.1 Common Failure Modes

| Failure Mode | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Forklift not detected (unseen model type) | Medium | Critical | Diverse forklift training data, "vehicle_other" catch-all class |
| Person partially behind vehicle | High | High | Tracking maintains ID through brief occlusion |
| Ground not flat (ramps, multi-level) | Medium | High | Per-zone calibration, warn operator of limitation |
| Homography drift (camera bumped/moved) | Low | Critical | Periodic auto-calibration check using reference points |
| False proximity from stacked goods | Medium | Medium | Height-based filtering — detected objects below expected vehicle height are likely goods |
| Reflections on shiny floors | Low | Medium | Include reflective surface data in training |

### 8.2 Calibration Risks

Homography assumes a flat ground plane. In environments with ramps, stairs, or multi-level areas, the distance estimation degrades significantly.

Mitigation: Define separate calibration zones for different ground levels. Alert the operator if a camera covers areas with > 1 meter elevation change.

### 8.3 Tracking Failure Risks

In crowded scenes (> 15 people + 3 vehicles), ByteTrack may experience ID switches.

Mitigation: Use appearance features (Re-ID) in addition to IoU matching. BoT-SORT is a viable alternative with built-in Re-ID.

Proximity alerts should be based on instantaneous distance (per-frame) rather than requiring tracking consistency — even if the track ID switches, the proximity danger is still detected.

## 9. Final Performance Targets

### 9.1 Model Performance

| Metric | Minimum | Target | Best-in-Class |
| --- | --- | --- | --- |
| mAP@0.5 (vehicle classes) | 85.0% | 91.0% | 95.0% |
| mAP@0.5 (person class) | 90.0% | 95.0% | 98.0% |
| Distance Estimation MAE | ±1.5 m | ±0.8 m | ±0.3 m |
| Proximity Alert Accuracy | 80.0% | 88.0% | 95.0% |
| Tracking MOTA | 60.0% | 75.0% | 85.0% |

### 9.2 Operational Performance

| Metric | Minimum | Target | Best-in-Class |
| --- | --- | --- | --- |
| Total Pipeline Latency | < 100 ms | < 50 ms | < 30 ms |
| False Proximity Alarms | < 5/hr | < 2/hr | < 0.5/hr |
| Missed DANGER events | < 10% | < 5% | < 2% |
| Throughput (FPS) | ≥ 10 | ≥ 15 | ≥ 25 |

### 9.3 Deployment Targets

| Parameter | Target |
| --- | --- |
| Model Format | PyTorch (dev) / TensorRT FP16 (production) |
| Additional Compute | ByteTrack tracker + homography (< 5 ms) |
| Calibration Required | Yes — one-time per camera (production) |
| Model Size | < 25 MB (independent model) |
| Dev Hardware | Laptop/Desktop with CUDA GPU |
| Edge Hardware | NVIDIA Jetson Orin Nano (production) |

End of Model 2 Specification — Vehicle Proximity Risk Detection