# Model Specification 3 — Fall Detection

Document Version:  1.0

Date:  2026-02-18

Author:  VisionSafe360 AI Research Team

Classification:  Internal Technical Specification

## 1. Model Objective

### 1.1 Problem Statement

Falls are the leading cause of death in the construction industry and the second-leading cause of workplace fatalities overall (Bureau of Labor Statistics). In 2022, 700 workers died from falls in the U.S. alone. Beyond falls from heights (ladders, scaffolding), slip-and-fall incidents on the same level account for 27% of all workplace injuries (National Safety Council). Rapid detection of a fall event enables immediate medical response, which is critical — in cardiac arrest or head trauma cases, survival rates decrease by 10% for every minute without intervention.

### 1.2 Detection Scope

The model must detect when a person transitions from an upright/normal posture to a fallen state.

Fall Event Types:

| Type | Description | Severity |
| --- | --- | --- |
| Sudden Collapse | Person collapses vertically (cardiac, heatstroke) | CRITICAL |
| Trip/Slip Fall | Person trips and falls to the ground | HIGH |
| Gradual Fall | Person slowly slumps to the ground (fatigue) | HIGH |
| Fall from Height | Person falls from elevated position | CRITICAL |

Non-Fall Events (must NOT trigger alert):

| Event | Description |
| --- | --- |
| Bending Down | Person bends to pick up an object |
| Sitting | Person sits on a chair, crate, or ground |
| Lying Intentionally | Worker resting during break in designated area |
| Crawling | Worker crawling in confined space |
| Kneeling | Worker kneeling to perform a task |

### 1.3 Technical Challenge

Fall detection is fundamentally a temporal event recognition problem, not a single-frame classification. The challenge is distinguishing between:

Intentional transitions to ground level (bending, sitting, kneeling)

Involuntary falls (sudden, uncontrolled descent)

This requires analyzing the dynamics of the person's movement — speed of descent, body orientation change rate, post-fall immobility — not just a single frame.

### 1.4 Business Impact

Immediate emergency response (< 30 seconds from fall to alert) can be life-saving.

Regulatory requirement in many jurisdictions for lone worker monitoring.

Insurance premium reduction for facilities with automated fall detection.

## 2. Industry Benchmark Research

### 2.1 Approach Taxonomy

Fall detection methods are broadly categorized into three families:

| Category | Description | Pros | Cons |
| --- | --- | --- | --- |
| Pose-based | Use skeleton keypoints to analyze body geometry | Interpretable, robust | Requires accurate pose estimation |
| Appearance-based | CNN classifies frames as fall/no-fall | Simple pipeline | Poor temporal reasoning |
| Spatiotemporal | 3D CNNs or RNNs analyzing video clips | Best accuracy | High compute, latency |

### 2.2 Academic State-of-the-Art

| Study | Year | Approach | Accuracy | Precision | Recall | F1 | Dataset |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tran et al., "SlowFast for Fall Detection" | 2023 | SlowFast R50 (3D CNN) | 97.8% | 96.2% | 97.1% | 96.6% | UR Fall |
| Chen et al., "YOLO-Pose + LSTM Fall" | 2023 | Pose estimation + LSTM sequence | 96.5% | 95.1% | 97.8% | 96.4% | Multiple datasets |
| Kong et al., "Skeleton-based Fall Detection" | 2022 | ST-GCN on keypoints | 97.2% | 95.8% | 96.9% | 96.3% | NTU RGB+D 120 |
| Li et al., "Lightweight Fall Detection" | 2023 | MoveNet + rule-based + CNN verify | 94.1% | 93.0% | 95.2% | 94.1% | Custom industrial |
| Wang et al., "Two-stage Fall Detector" | 2022 | YOLOv5 + aspect ratio + velocity | 93.7% | 91.5% | 96.0% | 93.7% | Le2i + custom |

### 2.3 Key Insights from Literature

Pose-based methods achieve the best balance of accuracy and interpretability.

Temporal analysis is essential — single-frame methods suffer from high false positive rates (sitting/bending classified as falls).

Two-stage approaches (detect person → analyze fall) outperform single-stage on real-world data.

The "lying on ground" verification after initial fall detection reduces false positives by 30–50%.

Real-world accuracy drops 5–15% compared to lab datasets due to occlusion, varied clothing, and camera angles.

### 2.4 Fall Detection Datasets

| Dataset | Samples | Environment | Simulated/Real | Public |
| --- | --- | --- | --- | --- |
| UR Fall Detection | 70 | Laboratory | Simulated | Yes |
| Le2i Fall | 221 | Home/office | Simulated | Yes |
| MCFD (Multi-camera) | 24 seq | Nursing home | Simulated | Yes |
| UP-Fall | 850 | Laboratory | Simulated | Yes |
| NTU RGB+D 120 | 114,480 | Laboratory | Simulated | Yes |
| High-Quality Fall | 520 | Varied | Real + Sim | Partial |

## 3. Recommended Model Architecture

### 3.1 Approach Comparison for Edge Deployment

| Approach | Edge Latency | Accuracy | False Positive Rate | Complexity |
| --- | --- | --- | --- | --- |
| SlowFast 3D CNN | ~200 ms | Very High | Low | Very High |
| ST-GCN on keypoints | ~100 ms | Very High | Low | High |
| Pose + LSTM | ~60 ms | High | Medium | Medium |
| Pose + Rule-based | ~40 ms | Medium | Medium-High | Low |
| Pose + Rule + CNN verify | ~55 ms | High | Low | Medium |
| Bounding box heuristics only | ~5 ms | Low | High | Very Low |

### 3.2 Recommended Architecture: Two-Stage Pose-Based Fall Detection

Stage 1: Real-Time Fall Candidate Detection (every frame)

Person Detection (YOLOv8s) → Pose Estimation (YOLO-Pose) → Geometric Rule Engine → Fall Candidate

The geometric rule engine analyzes per-frame pose features:

| Feature | Description | Fall Indicator |
| --- | --- | --- |
| Bounding Box Aspect Ratio | Width/Height of person bbox | > 1.0 (wider than tall) |
| Hip-to-Ground Height | Normalized Y-coordinate of hip keypoint | < 0.3 of initial height |
| Torso Angle | Angle of shoulder-hip vector relative to vertical | > 60° from vertical |
| Head Position | Y-coordinate of head relative to body bbox | Below hip level |
| Vertical Velocity | Frame-to-frame descent rate of centroid | > threshold px/frame |

Stage 2: Temporal Verification (triggered by Stage 1)

When Stage 1 flags a fall candidate, a temporal buffer of the last 1–2 seconds of pose data is analyzed:

| Temporal Feature | Description |
| --- | --- |
| Descent Duration | Time from upright to ground-level |
| Post-Fall Immobility | Person remains in ground position for > 3 seconds |
| Velocity Profile | Sudden spike → deceleration (vs. gradual descent) |
| Pre-Fall Posture | Was person upright immediately before? |

Decision Logic:

IF (aspect_ratio > 1.0) AND (hip_height < threshold) AND (descent_velocity > threshold): fall_candidate = True IF fall_candidate AND (immobile_duration > 3 seconds): fall_confirmed = True → Trigger CRITICAL alert IF fall_candidate AND (person resumes upright within 3 seconds): fall_dismissed → Log as "stumble" event, WARNING alert

### 3.3 Pose Estimation Model Selection

| Model | Keypoints | Latency (Jetson) | AP (COCO) | Edge Suitability |
| --- | --- | --- | --- | --- |
| YOLO-Pose (v8s) | 17 | ~35 ms | 69.0 | Excellent |
| MoveNet Lightning | 17 | ~12 ms | 53.0 | Good |
| MoveNet Thunder | 17 | ~30 ms | 64.0 | Good |
| MediaPipe Pose | 33 | ~25 ms | N/A | Good |
| HRNet-W32 | 17 | ~120 ms | 74.4 | Poor |
| ViTPose-S | 17 | ~80 ms | 73.8 | Moderate |

Recommendation: YOLO-Pose (YOLOv8s-pose)

Rationale:

Single model performs both person detection AND pose estimation (no separate detector needed).

Shares the same YOLOv8s backbone architecture as Models 1 and 2.

Can potentially share early backbone features if models are unified.

17 COCO keypoints include all needed joints (ankles, knees, hips, shoulders, head).

TensorRT-compatible for Jetson deployment.

### 3.4 Multi-Person Fall Detection

The system must handle multiple people simultaneously:

YOLO-Pose natively detects multiple persons with individual pose estimates.

ByteTrack assigns persistent IDs to maintain per-person fall state.

Each tracked person has an independent fall-state machine: UPRIGHT → FALL_CANDIDATE → FALL_CONFIRMED → FALL_RESOLVED.

## 4. Dataset Strategy

### 4.1 Dataset Requirements

Unlike object detection, fall detection requires video sequences, not single images:

| Data Type | Quantity | Purpose |
| --- | --- | --- |
| Fall event videos | 500 sequences | Positive training samples |
| Normal activity videos | 2,000 sequences | Negative samples |
| Confusing activity videos | 500 sequences | Hard negatives |
| Industrial environment footage | 300 sequences | Domain adaptation |
| Total | ~3,300 seq |  |

### 4.2 Confusing Activities (Hard Negatives)

These are critical for reducing false positives:

| Activity | Frequency in Workplace | Fall Confusion Risk |
| --- | --- | --- |
| Bending to pick up | Very High | Medium |
| Sitting on ground | Medium | High |
| Kneeling at machine | High | Medium |
| Lying down (break) | Low | Very High |
| Climbing ladder | Medium | Low |
| Pushing heavy load | Medium | Low |
| Stretching | Low | Medium |

### 4.3 Data Collection Strategy

Phase 1: Public Dataset Compilation

Combine UR Fall, Le2i, UP-Fall datasets for initial training.

Extract person pose sequences using YOLO-Pose.

Phase 2: Simulated Industrial Falls

Record 200+ simulated fall events on-site with volunteers.

Include variations: forward fall, backward fall, sideways fall, collapse.

Record from the exact camera angles and heights used in deployment.

Phase 3: Normal Activity Recording

Record 1,000+ normal work activity sequences from deployment cameras.

Label activities: walking, bending, sitting, kneeling, carrying, etc.

Phase 4: Continuous Learning

After deployment, flag all false positives/negatives for human review.

Retrain quarterly with corrected labels.

### 4.4 Pose Data Augmentation

| Augmentation | Description |
| --- | --- |
| Temporal Speed Jitter | Speed up / slow down fall sequences (0.5x–2.0x) |
| Keypoint Noise | Add Gaussian noise to keypoint positions (σ=3px) |
| Mirror | Horizontal flip of all sequences |
| Camera Angle Simulation | Adjust keypoint positions for different angles |
| Partial Occlusion | Randomly drop 2–3 keypoints per frame |

## 5. Training Strategy

### 5.1 Two-Component Training

Component A: Pose Estimation (YOLO-Pose)

Use pre-trained YOLOv8s-pose weights (trained on COCO).

Fine-tune on industrial environment images for 20–30 epochs.

Focus on improving pose accuracy for unusual body positions (bent, crouching, on ground).

Component B: Fall Classifier (if using learned approach)

Input: Sequence of 30–60 frames of normalized pose keypoints (17 joints × 2 coordinates × T frames).

Architecture: Lightweight 1D-CNN or 2-layer LSTM.

Binary classification: fall / not-fall.

Train on pose sequences extracted from video datasets.

### 5.2 Rule-Based vs. Learned Classifier

| Aspect | Rule-Based | Learned (LSTM/1D-CNN) |
| --- | --- | --- |
| Training data needed | None (hand-tuned) | 1,000+ sequences |
| Interpretability | Full | Partial |
| Accuracy | 88–92% | 94–97% |
| False positive rate | 5–10% | 2–5% |
| Edge compute | Negligible | 3–10 ms |
| Adaptability | Manual threshold tuning | Retraining |

Recommendation: Start with rule-based, upgrade to learned classifier in V2.

The rule-based approach is faster to deploy, fully interpretable (critical for safety certification), and can be tuned on-site without retraining. Once sufficient real-world data is collected (3–6 months of deployment), train a lightweight LSTM classifier for improved accuracy.

### 5.3 Threshold Calibration

The rule-based system has tunable thresholds that should be calibrated per-site:

| Threshold | Default Value | Calibration Method |
| --- | --- | --- |
| Aspect Ratio (fall) | > 1.0 | Adjust for camera angle |
| Hip Height (normalized) | < 0.30 | Measure average person height |
| Vertical Velocity | > 15 px/frame | Scale with camera FPS and resolution |
| Immobility Duration | > 3 seconds | Adjust for industry (shorter for high-risk zones) |
| Torso Angle (from vertical) | > 60° | Consistent across sites |

### 5.4 Training Epochs & Schedule (for Learned Classifier)

| Phase | Epochs | Learning Rate | Description |
| --- | --- | --- | --- |
| Initial Training | 50 | 1e-3 | Train on public datasets |
| Fine-Tuning | 30 | 1e-4 | Fine-tune on site data |
| Continuous Update | 10 | 5e-5 | Quarterly retraining |

## 6. Evaluation Metrics

### 6.1 Primary Metrics

| Metric | Definition | Priority |
| --- | --- | --- |
| Sensitivity (Recall) | % of actual falls correctly detected | CRITICAL |
| Specificity | % of non-fall events correctly classified | HIGH |
| False Alarm Rate | Number of false fall alerts per camera per day | HIGH |
| Detection Latency | Time from fall onset to alert | HIGH |

### 6.2 Confusion Matrix Targets

|  | Predicted: Fall | Predicted: No Fall |
| --- | --- | --- |
| Actual: Fall | TP (> 95%) | FN (< 5%) |
| Actual: No Fall | FP (< 3%) | TN (> 97%) |

### 6.3 Operational Metrics

| Metric | Definition | Target |
| --- | --- | --- |
| Mean Detection Time | Average seconds from fall onset to alert | < 5 seconds |
| Maximum Detection Time | Worst-case fall detection latency | < 10 seconds |
| False Alarms Per Camera/Day | Number of false fall alerts per day | < 5 |
| Fall Detection Rate (24h) | % of falls detected during day AND night | > 95% day, > 85% night |

### 6.4 Performance Targets

| Metric | Minimum | Target | Best-in-Class |
| --- | --- | --- | --- |
| Sensitivity (Fall Recall) | 90.0% | 95.0% | 98.0% |
| Specificity | 92.0% | 97.0% | 99.0% |
| F1 Score | 90.0% | 95.0% | 97.0% |
| Detection Latency | < 10 s | < 5 s | < 3 s |
| False Alarms per cam/day | < 10 | < 5 | < 1 |

## 7. Deployment Strategy

### 7.0 Development vs. Production Environments

Same laptop-first workflow as Models 1 and 2. All training, testing, and demos run on a laptop/desktop GPU. Edge deployment is the future production target.

| Phase | Hardware | Model Format | Notes |
| --- | --- | --- | --- |
| Development | Laptop/Desktop (CUDA GPU) | PyTorch (.pt) | Full development, training, testing |
| Demo/Testing | Laptop/Desktop | PyTorch or ONNX | Real-time demo with webcam or video |
| Production | NVIDIA Jetson Orin Nano | TensorRT FP16 | Final edge deployment (future phase) |

### 7.1 Pipeline Latency Budget

| Component | Latency |
| --- | --- |
| YOLOv8s-Pose inference (TensorRT) | ~35 ms |
| ByteTrack update | ~3 ms |
| Geometric feature extraction | < 1 ms |
| Rule-based fall decision | < 1 ms |
| Total per frame | ~40 ms |

The temporal verification (immobility check) operates over accumulated state, not per-frame — no additional compute.

### 7.2 Model Export

| Format | Size | Latency (Jetson Orin Nano) |
| --- | --- | --- |
| PyTorch (.pt) | ~25 MB | ~80 ms |
| ONNX | ~22 MB | ~55 ms |
| TensorRT FP16 | ~14 MB | ~35 ms |

### 7.3 State Machine on Edge

Each tracked person maintains a fall state machine on-device:

UPRIGHT ──(fall indicators triggered)──► FALL_CANDIDATE FALL_CANDIDATE ──(immobile > 3s)──► FALL_CONFIRMED ──► Alert triggered FALL_CANDIDATE ──(person stands up)──► UPRIGHT (log as "stumble") FALL_CONFIRMED ──(manual reset by operator)──► UPRIGHT

State transitions are logged and sent to the backend as HazardEvent instances.

### 7.4 Night/Low-Light Considerations

Fall detection must work 24/7. Strategies:

| Challenge | Mitigation |
| --- | --- |
| Low ambient light | IR-compatible cameras (most industrial CCTV) |
| Poor pose estimation | Increase keypoint confidence threshold |
| Shadows/artifacts | Train on low-light augmented data |
| Thermal cameras | Fine-tune pose model on thermal imagery (V2) |

## 8. Risks & Mitigation

### 8.1 Risk Matrix

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Bending classified as fall (false alarm) | High | Medium | Temporal verification (3s immobility requirement) |
| Person sitting on ground triggers alert | Medium | Medium | Designated rest zone exclusion, gradual descent filter |
| Occluded person — missed keypoints | High | High | Fall back to bounding box aspect ratio analysis |
| Night / poor lighting reduces accuracy | Medium | High | IR cameras, low-light training augmentation |
| Slip-and-recovery missed (brief fall) | Medium | Low | Log as "stumble" event, lower severity alert |
| Person already on ground when camera starts | Low | Medium | Initial state assumes standing, ignore static poses |
| Multiple people occluding each other | Medium | High | Per-person tracking, require minimum keypoint count |
| Camera angle too steep (top-down view) | Low | High | Vertical velocity less meaningful; use area change |

### 8.2 Critical Safety Considerations

This is a life-safety system. False negatives (missed falls) are categorically worse than false positives (false alarms).

The system should never be the sole fall protection measure — it supplements, not replaces, physical safety systems (guardrails, harnesses, buddy systems).

Alert escalation: if no operator acknowledgment within 60 seconds of a FALL_CONFIRMED alert, automatically escalate to emergency contacts and/or trigger audible siren.

### 8.3 Liability & Certification

Document all detection capabilities and limitations clearly for facility operators.

Do NOT market as "100% fall detection" — state measured sensitivity and conditions tested.

Include clear disclaimer that the system is an aid, not a replacement for safety protocols.

Consider IEC 62443 (industrial cybersecurity) and ISO 13849 (safety of machinery) compliance paths.

## 9. Final Performance Targets

### 9.1 Model Performance

| Metric | Minimum | Target | Best-in-Class |
| --- | --- | --- | --- |
| Fall Sensitivity (Recall) | 90.0% | 95.0% | 98.0% |
| Specificity | 92.0% | 97.0% | 99.0% |
| F1 Score | 90.0% | 95.0% | 97.0% |
| Detection Latency | < 10 s | < 5 s | < 3 s |
| False Alarms per cam/day | < 10 | < 5 | < 1 |

### 9.2 Operational Performance

| Metric | Minimum | Target | Best-in-Class |
| --- | --- | --- | --- |
| Pipeline Latency | < 80 ms | < 45 ms | < 30 ms |
| 24/7 Uptime | 95% | 99% | 99.9% |
| Night Sensitivity | 80% | 90% | 95% |
| Throughput (FPS) | ≥ 10 | ≥ 20 | ≥ 30 |

### 9.3 Deployment Targets

| Parameter | Target |
| --- | --- |
| Pose Model | YOLOv8s-Pose (TensorRT FP16) |
| Fall Classifier (V1) | Rule-based geometric + temporal engine |
| Fall Classifier (V2) | Lightweight LSTM (< 1 MB) |
| Combined Model Size | < 15 MB |
| State per Person | ~100 bytes (keypoint history buffer) |
| Dev Hardware | Laptop/Desktop with CUDA GPU |
| Edge Hardware | NVIDIA Jetson Orin Nano (production) |

End of Model 3 Specification — Fall Detection