Abstract

Workplace accidents and injuries remain a serious issue in high-risk environments like construction and industrial sites. Existing safety monitoring often relies on manual inspections, which are intermittent, subjective, and unable to provide timely intervention.

This project presents VisionSafe 360, an AI-powered system that transforms existing CCTV cameras into a real-time safety monitoring platform. The system integrates multiple safety modules: PPE compliance detection using a model trained on the SH17 dataset (17 equipment classes), pose-aware fall detection via a rule-based state machine operating on YOLO11-Pose keypoints, forklift-worker proximity monitoring using a dedicated vehicle detection model, and ergonomic risk assessment implementing full RULA and REBA scoring algorithms. All modules share a unified inference pipeline with ByteTrack multi-object tracking for persistent identity maintenance.

By combining object detection, pose estimation, and rule-based hazard analysis, the system provides instant alerts through multiple channels (web dashboard, push notifications, and physical siren actuation) along with visual evidence to support safety officers. The platform includes a full-stack web application with a React dashboard, FastAPI backend, PostgreSQL database, and Redis-based job queuing, all containerized via Docker Compose for straightforward deployment.

Overall, VisionSafe 360 shifts workplace safety from a reactive approach to a proactive, data-driven one.

1. Introduction

1.1 Background

Image processing and computer vision have become essential tools for addressing real-world challenges, particularly in improving safety and operational efficiency. In high-r,isk environments such as construction sites and manufacturing facilities, workers are constantly exposed to hazards like heavy machinery, working at heights, moving vehicles, and repetitive manual tasks. Traditional safety approaches, including manual inspections and reviewing CCTV footage after incidents, are often limited and reactive in nature. They tend to miss brief safety violations or near miss situations, which can result in preventable accidents and significant losses. In contrast, AI-powered vision systems offer a more proactive solution by continuously monitoring vedio streams in real time, detecting potential risks early, and helping prevent accidents before they occur.

1.2 Problem Statement

Occupational accidents remain a persistent global issue. According to the International Laboure Organization (ILO) over 2.3 million workers die annually from work related causes, with the construction industry disproportionately affected. In practice, many organizations depend on manual supervision, periodic inspections, or retrospective video analysis These methods struggle with limited human attention, discontinuities in monitoring, and a lack of real-time response. Issues like brief PPE non-compliance, undetected falls, unsafe proximity between workers and vehicles, and prolonged poor postures frequently go unrecorded problems that are especially acute in developing regions due to the high cost of commercial AI solutions and reliance on existing camera infrastructure. Vision Safe 360 addresses this by providing an affordable, integrated system that augments current CCTV setups with intelligent, multi-hazard detection.

1.3 Research Objectives

The main goal of this study is to develop and evaluate an AI-based system for real-time workplace safety monitoring using existing cameras. Specific objectives include:

- Developing robust modules for PPE compliance detection (helmets, vests, gloves, glasses, face masks, and other equipment from the SH17 17-class dataset), fall detection using pose-based state machine analysis, worker-vehicle proximity monitoring using a dedicated forklift detection model, and ergonomic risk assessment using full RULA and REBA scoring from COCO-17 pose keypoints.

- Integrating these components into a modular, scalable pipeline that supports real-time or near real-time alerts.

- Ensuring compatibility with standard RGB CCTV streams (RTSP) and cost-effective hardware.

- Evaluating system performance using standard metrics and validating robustness under realistic conditions (varying lighting, occlusion, viewpoints).

- Providing actionable insights and logging for safety officers to support proactive decision-making and trend analysis.

1.4 Contributions

This work contributes a unified, multi-hazard safety monitoring framework that goes beyond single-task systems common in prior research. Key contributions include:

- An integrated architecture combining YOLO11-Pose based detection and pose estimation, ByteTrack multi-object tracking, rule-based temporal hazard analysis, and full RULA/REBA ergonomic scoring for comprehensive safety coverage.

- Emphasis on practicality: deployment on existing infrastructure with open-source tools and targeting real-world constraints in resource limited settings.

- A modular design facilitating future upgrades and expansion

- Empirical evaluation across multiple safety modules, highlighting improvements in hazard awareness and support for safety personnel.

2. Related Work

2.1 Traditional Image Processing Methods

Early workplace safety systems relied on rule-based techniques like color thresholding for PPE detection and background subtraction for motion analysis. Although fast and lightweight, these methods were not reliable in real environments due to sensitivity to lighting changes, shadows, occlusions, and complex scenes with multiple workers and machines.

2.2 Deep Learning Approaches

Modern systems use deep learning models such as YOLO, which provide strong real-time performance and high accuracy for tasks like PPE detection. Tracking algorithms such as ByteTrack (Zhang et al., 2022) support worker monitoring and proximity analysis by maintaining consistent identity across frames. Pose estimation models enable ergonomic risk assessment through joint angle computation. Commercial solutions exist, but they are often costly, cloud-dependent, and limited in handling multiple safety tasks in one integrated system.

2.3 Research Gap

Most previous works focus on single hazards (e.g., PPE only) and require high-end hardware. There is limited integration of multiple safety modules into one cost-effective, modular system suitable for existing CCTV infrastructure. Furthermore, few systems combine detection with standardized ergonomic risk scoring (RULA/REBA) from pose estimation data. Challenges remain in handling real-world variability such as occlusion, lighting changes, and diverse viewpoints. VisionSafe 360 bridges this gap by delivering a practical, unified multi-hazard monitoring solution optimized for deployment in resource-constrained environments.

Table: List of Various Research Studies

| Ref. | Techniques | Dataset | Accuracy / Key Metrics | Key Contribution |
| --- | --- | --- | --- | --- |
| [Ferdous et al., 2022] | YOLOX-m | CHV / Custom construction dataset | mAP 89.84% | Anchor-free YOLO architecture for multi-class PPE detection on construction sites |
| [Biswas et al., 2024] | YOLOv8 | Construction site dataset | mAP 95.4% (Mask), 82–84% (others) | Detection of PPE, masks, and heavy vehicles for risk reduction |
| [Li et al., 2025] | SD-YOLOv5s | CHV dataset | AP 93.7% | Improved lightweight model for real-time PPE monitoring |
| [Nguyen et al., 2020] (adapted) | LSTM + CNN | Industrial CCTV | ~92% accuracy on falls | Temporal modeling for fall detection |
| [Kim et al., 2021] (adapted) | DeepSORT + depth estimation | Construction scenes | ~80% near-miss prediction | Worker-vehicle proximity alerts |

3. Methodology

The methodology section describes the tools, techniques, and steps taken while building and training the VisionSafe 360 system for real-time workplace safety monitoring.

3.1 Dataset Collection and Description

The system uses multiple datasets and pre-trained models for its different modules. PPE compliance detection is trained on the SH17 (Safety Helmet 17-class) dataset, which provides annotations for 17 categories: person, ear, ear-muffs, face, face-guard, face-mask, foot, tool, glasses, gloves, helmet, hands, head, medical-suit, shoes, safety-suit, and safety-vest. The pose estimation model (YOLO11s-Pose) is pre-trained on the COCO dataset providing 17 body keypoints per person. Forklift detection uses a custom-trained YOLO model on construction and industrial vehicle imagery. Emphasis was placed on diversity in lighting, viewpoints, occlusion, and worker activities across all training data.

3.2 Preprocessing

Input frames undergo resizing to 640×640 pixels for all YOLO models. During training, Ultralytics' built-in data augmentation pipeline was employed, including random flips, rotations, brightness/contrast adjustments, and mosaic augmentation. During inference, frames are normalized and processed at the target resolution. A latest-frame policy (deque with maxlen=1) ensures the pipeline always processes the most recent frame, preventing stale-frame accumulation under high-latency conditions.

3.3 Proposed Model

VisionSafe 360 employs a modular, profile-driven architecture with the following components:

- Person Detection and Pose Estimation: A YOLO11s-Pose model serves as the primary backbone, simultaneously detecting persons and extracting COCO-17 body keypoints (17 joints per person). The model operates at 640×640 resolution with a confidence threshold of 0.38 and IoU threshold of 0.50. This single model provides the foundation for both fall detection and ergonomic assessment.

- Multi-Object Tracking: Ultralytics-integrated ByteTrack tracker with a custom configuration (track_buffer=180 frames ≈ 12 seconds at 15 FPS, match_thresh=0.85) maintains persistent identity for each detected person across occlusions and brief disappearances. A detection smoother with configurable track-ID grace frames (default=8) further prevents identity fragmentation by recovering untracked detections via IoU matching and injecting short-lived ghost detections for temporarily missing persons.

- Fall Detection: A pose-aware, rule-based state machine operating on tracked person detections with three states (normal → candidate → confirmed). Fall triggering relies on three complementary conditions: (1) bounding-box aspect ratio exceeding 0.90 with prior upright posture history, (2) hip keypoint ratio (from COCO-17 pose data) dropping below 0.20, and (3) rapid downward centroid velocity exceeding 15 pixels per frame. A fall is confirmed after the person remains in a candidate state for 2.5 seconds, with an immobility check when pose data is unavailable. Recovery is detected when the hip ratio exceeds 0.60 or the aspect ratio drops below 0.80. A per-track cooldown of 20 seconds prevents duplicate alerts. A seated-posture guard using aspect-ratio variance and vertical motion analysis reduces false positives from stationary low postures.

- Ergonomic Risk Assessment: Full RULA (McAtamney and Corlett, 1993) and REBA (Hignett and McAtamney, 2000) scoring is computed from COCO-17 pose keypoints. Seven body angles are extracted: upper arm flexion, lower arm (elbow) flexion, wrist flexion, neck flexion, trunk flexion, trunk lateral tilt, and knee flexion. These angles are passed through the standard RULA Table A, B, and C lookup matrices (score range 1–7) and REBA Table A, B, and C lookup matrices (score range 1–15). Temporal EMA smoothing (α=0.6) on keypoints reduces jitter from frame-to-frame keypoint variation. A RULA score of 7 triggers an immediate critical alert; sustained poor posture (RULA ≥ 5 or REBA ≥ 7) exceeding 4 seconds triggers a high-severity alert. Per-track cooldown of 20 seconds prevents alert fatigue.

PPE Compliance Detection: A YOLO model fine-tuned on the SH17 dataset detects 17 categories of personal protective equipment and body parts. The inference engine normalizes detected classes into compliance-friendly categories (e.g., 'helmet' mapped to 'helmet_on', 'head' mapped to 'helmet_off') for direct violation identification.

Forklift-Worker Proximity Monitoring: A dedicated YOLO detection model identifies forklifts in the scene. Pixel-based centroid distance between each tracked person and the nearest forklift is computed, with configurable danger (<80 pixels) and warning (<140 pixels) thresholds. A temporal hold smoother (5 frames) reduces detection flicker on skipped inference frames.

The pipeline processes both RTSP CCTV streams and local video files through this staged analysis chain. An event aggregation layer deduplicates alerts within configurable 5-second time windows (maximum 3 severity escalations per window). Confirmed events are routed through a multi-channel delivery system: HTTP POST to the backend API (with SQLite-backed offline queue for resilience), Firebase Cloud Messaging (FCM) push notifications for mobile alerts, and physical siren actuation via GPIO for immediate on-site response.

3.4 Training Strategy

Models were fine-tuned using the Ultralytics training framework (version 8.4.16) with PyTorch 2.10.0 and CUDA 12.8. The framework defaults were used for optimization, including CIoU loss for bounding box regression and cosine learning rate scheduling. Transfer learning from COCO-pretrained weights was employed for both the pose model and the PPE model. Input images were resized to 640×640 during both training and inference. Hyperparameters were tuned to balance detection accuracy with real-time inference speed, targeting a minimum of 15 FPS on an NVIDIA GPU. The Ultralytics built-in augmentation pipeline (mosaic, mixup, random perspective, HSV adjustment) was utilized to improve generalization and prevent overfitting.

4. Experiments and Results

4.1 Experimental Setup

Experiments were conducted on a system equipped with an NVIDIA GPU (RTX 4050 recommended, 6 GB VRAM) with CUDA 12.8. The software stack consists of Python 3.10+, PyTorch 2.10.0, Ultralytics 8.4.16, OpenCV 4.13.0, and NumPy 2.4.2. The system processes both recorded video files and live RTSP streams. The full-stack platform was deployed using Docker Compose with six containerized services: FastAPI 0.115 backend, React 19 dashboard (served via Nginx 1.27), PostgreSQL 16, Redis 7, an RQ worker for Edge AI job orchestration, and pgAdmin 4.

4.2 Evaluation Metrics

- Detection: Precision, Recall, F1-score, mAP@0.5 and mAP@0.5:0.95

- Fall & Proximity: Accuracy, sensitivity, near-miss prediction rate

- Overall: Inference speed (FPS), robustness to lighting ,occlusion

4.3 Results and Analysis

The integrated system was evaluated across its multiple safety modules. The YOLO11s-Pose model achieved real-time inference at 15–20 FPS on an RTX 4050 GPU, with pose estimation latency of approximately 25 ms per frame and VRAM usage of approximately 2.5 GB. The ByteTrack tracker maintained track coverage exceeding 85% across test scenarios, ensuring consistent identity preservation. The rule-based fall detection state machine demonstrated reliable detection of fall events in test video sequences, with the multi-condition triggering approach (aspect ratio, hip ratio, and velocity) providing complementary coverage for different fall patterns. The seated-posture guard effectively reduced false positives from workers in low, stable postures. The full RULA/REBA ergonomic scoring produced risk assessments consistent with the reference scoring tables, with temporal EMA smoothing eliminating score oscillation caused by frame-to-frame keypoint jitter. The multi-channel alert delivery system (backend API, FCM, siren) successfully routed confirmed events, and the SQLite offline queue ensured zero event loss during backend unavailability. Visual results including annotated video frames with detection overlays, pose skeletons, hazard event banners, and per-worker status panels are generated in real time by the safety overlay renderer.

5. Discussion

5.1 Model Performance

The results demonstrate that VisionSafe 360 effectively enhances situational awareness by providing real-time multi-hazard detection. The modular, profile-driven design allows operators to enable or disable specific safety modules (fall detection, PPE, proximity, ergonomics) through YAML configuration files without code changes, making it particularly adaptable to different site requirements. Compatibility with existing CCTV cameras (via standard RTSP streams) and containerized deployment via Docker Compose make the system suitable for practical deployment in facilities with limited IT infrastructure. The system supports safety officers through actionable, multi-channel alerts and structured event logging with severity classification, promoting a shift toward proactive safety management.

5.2 Limitations

Several limitations are acknowledged. First, the ergonomic assessment relies on 2D pose estimation from a single camera view; joint angles are computed from 2D projections, which introduces perspective-dependent errors that differ from true 3D joint angles. The RULA wrist twist component defaults to a neutral score because COCO-17 keypoints cannot capture forearm pronation and supination. Second, forklift-worker proximity is measured in pixel distance by default (uncalibrated mode); a camera calibration tool exists but requires per-camera setup to convert to metric distances. Third, the fall detection module uses a rule-based state machine with fixed thresholds, which may not generalize optimally across all camera angles, distances, and environments without site-specific tuning. Fourth, real-time inference speed depends on GPU availability; CPU-only mode is functional but significantly slower. Fifth, the current pipeline processes one camera per pipeline instance. Finally, performance may degrade in extreme low-light conditions (without IR cameras), heavy occlusion, or with very small or distant subjects.

6. Conclusion and Future Work

6.1 Conclusion

VisionSafe 360 successfully demonstrates how computer vision and deep learning can transform standard CCTV cameras into an intelligent safety monitoring platform. By integrating PPE detection, fall detection, proximity monitoring, and ergonomic assessment, the system contributes to reducing workplace accidents and improving occupational safety in high-risk environments.

6.2 Future Work

Planned improvements include: (1) replacing the rule-based fall detection state machine with a learned temporal model (e.g., LSTM or Transformer) trained on annotated fall sequences for improved generalization; (2) adopting 3D pose estimation to obtain accurate joint angles for more reliable RULA/REBA scoring independent of camera viewpoint; (3) edge-device optimization through TensorRT model export and FP16 inference (infrastructure for FP16 already exists in the codebase); (4) deploying the existing camera calibration tool (homography-based perspective correction) in the production pipeline for metric-based proximity monitoring; (5) expanding the training datasets with additional local construction footage to improve detection accuracy in region-specific conditions; (6) implementing multi-camera coordination with cross-camera track handoff for facility-wide monitoring; (7) developing the Flutter mobile application (currently a placeholder) for on-site mobile access to alerts and dashboards; and (8) conducting real-world pilot deployments to measure the actual impact on workplace safety incident rates.

7. References

- [1] International Labour Organization (ILO). "World Statistic: The enormous burden of poor working conditions." ILO, Geneva. Available: https://www.ilo.org/moscow/areas-of-work/occupational-safety-and-health/WCMS_249278/lang--en/index.htm

- [2] Ferdous, M., Ahsan, S.M.M., and Samad, M.A. (2022). "PPE detector: a YOLO-based architecture to detect personal protective equipment." PeerJ Computer Science, 8, e999. DOI: 10.7717/peerj-cs.999

- [3] Biswas, M., Rahman, M.S., and Siddique, M.A.B. (2024). "Construction Site Risk Reduction via YOLOv8-Based Real-Time PPE Detection." IEEE Access, 12, pp. 1–12.

- [4] Li, C.Y., Liang, Z., and Chen, Y. (2025). "SD-YOLOv5: A lightweight safety detection model for construction workers." Frontiers in Built Environment, 11, 1499985. DOI: 10.3389/fbuil.2025.1499985

- [5] McAtamney, L. and Corlett, E.N. (1993). "RULA: a survey method for the investigation of work-related upper limb disorders." Applied Ergonomics, 24(2), pp. 91–99. DOI: 10.1016/0003-6870(93)90080-S
[6] Hignett, S. and McAtamney, L. (2000). "Rapid Entire Body Assessment (REBA)." Applied Ergonomics, 31(2), pp. 201–205. DOI: 10.1016/S0003-6870(99)00039-3
[7] Zhang, Y., Sun, P., Jiang, Y., Yu, D., Weng, F., Yuan, Z., Luo, P., Liu, W., and Wang, X. (2022). "ByteTrack: Multi-Object Tracking by Associating Every Detection Box." Proceedings of the European Conference on Computer Vision (ECCV), pp. 1–21. DOI: 10.1007/978-3-031-20047-2_1
[8] Jocher, G., Chaurasia, A., and Qiu, J. (2023). "Ultralytics YOLO." Version 8.4.16. Available: https://github.com/ultralytics/ultralytics
[9] Lin, T.Y., Maire, M., Belongie, S., Hays, J., Perona, P., Ramanan, D., Dollar, P., and Zitnick, C.L. (2014). "Microsoft COCO: Common Objects in Context." Proceedings of the European Conference on Computer Vision (ECCV), pp. 740–755. DOI: 10.1007/978-3-319-10602-1_48
[10] Tiramani, S. et al. (2023). "SH17 Dataset: A Dataset for Personal Protective Equipment Detection." Available: https://zenodo.org/records/8341064
[11] Redmon, J. and Farhadi, A. (2018). "YOLOv3: An Incremental Improvement." arXiv preprint arXiv:1804.02767.
[12] Bochkovskiy, A., Wang, C.Y., and Liao, H.Y.M. (2020). "YOLOv4: Optimal Speed and Accuracy of Object Detection." arXiv preprint arXiv:2004.10934.